BMV-71x changelog & Bluetooth Interface changelog

How to update?

Update using VictronConnect. This program is available for download on https://www.victronenergy.com/software/. Either the build-in Bluetooth Smart interface is used with VictronConnect or special hardware is required. Use either the VE.Direct to USB interface (part number ASS030530000) or the VE.Direct Bluetooth Smart dongle (ASS030536010).

What version do I have?

Firmware version is visible in the setup menu, item 61. And also visible in VictronConnect. The BMV-7xx Smart Bluetooth Interface version is only available in VictronConnect on the Product Info page (when  connected via Bluetooth).

Changelog BMV-7xx:

Note: the changelog of the BMV-7xx Smart Bluetooth Interface has been added to this document as well. It’s located directly after this changelog.

v4.19 – 9 April 2025 2024 (BMV-712, BMV-712 Rev2)

- Add setting 76 to reverse the shunt direction (signedness of the Current will swap i.e. positive current becomes negative and vice versa). This setting is protected by the “Settings lock”.

- Support battery capacity up to 32,500 Ah (prior limit was set to 9,999 Ah).

v4.16 – 23 January 2024 (BMV-712, BMV-712 Rev2, BMV-710H)

- Fix: preserve settings lock key when performing restore to defaults

v4.15 – 26 September 2023 (BMV-712, BMV-712 Rev2, BMV-710H)

- Add new feature “Settings lock”. This feature will protect critical settings by locking them with a password set via VictronConnect. Non-critical settings can still be changed while the lock is active. The following settings are considered non-critical:

  - Current threshold(7)

  - Time-to-go averaging period(8)

  - Synchronize SoC to 100%(10)

  - Alarm buzzer(32)

  - Low SoC alarm(33) & Clear low SoC alarm(34)

  - Low voltage alarm(35) & Clear low voltage alarm(36)

  - High voltage alarm(37) & Clear high voltage alarm(38)

  - Low starter voltage alarm(39) & Clear low starter voltage alarm(40)

  - High starter voltage alarm(41) & Clear high starter voltage alarm(42)

  - High temperature alarm(43) & Clear high temperature alarm(44)

  - Low temperature alarm(45) & Clear low temperature alarm(46)

  - Midpoint deviation alarm(47) & Clear midpoint deviation alarm(48)

  - Backlight intensity(49)

  - Backlight always on(50)

  - Scroll speed(51),

  - Main voltage display(52), Current display(53), Power display(54), Consumed Ah display(55), State of charge display(56), Time-to-go display(57), Starter voltage display(58), Temperature display (59), Mid-voltage display(60),

  - Clear History(63)

  - Lock setup(64)

  - Temperature unit (67)

  - Bluetooth mode(71)

  - Bluetooth name

  - Bluetooth pin code

  - Instant Readout

  - Firmware updates

Note: the “Settings lock” is only available if the Bluetooth interface is running firmware v2.47 or higher! In case the Bluetooth interface firmware is below v2.47, please update the Bluetooth interface via VictronConnect on phone/tablet.

Note 2: the “Settings lock” will also protect settings from being changed via the head unit. However, locking and unlocking of the “Settings lock” is only possible via the Bluetooth or VE.Direct interface and not via the head unit!

Note 3: although there’s some overlap between the “Settings lock” and the setting “Lock setup(64)”, there’s a distinct difference: “Lock setup(64)” will lock all settings on the head unit only while “Settings lock” will lock critical settings via all interfaces of the BMV.

v4.13 – 24 January 2023 (BMV-712, BMV-712 Rev2, BMV-710H)

- Fix: incorrect calculated state of charge (SOC) when no temperature sensor is connected while the auxiliary input is set to "Temperature". This issue was found in v4.12 and could in extreme cases result in a SOC that is always 1% (see “Known issue” for version v4.12 below).

v4.12 – 6 October 2022 (BMV-712, BMV-712 Rev2)

This version is available via VictronConnect v5.80 since 14 December 2022. It will not be available on Victron Professional due to the known issue below. This issue has been fixed in v4.13.

Changes:

- Add new feature “Keep SOC on reset”. This feature will remember the state of charge during a power loss. This fixes the problem that, after the system has been switched off, the BMV would need to be fully recharged to resync and show a state of charge again. This feature will remember the state of charge, and - besides battery self discharge which in most cases will be negligible - will make for a reliable state of charge reading also when shutting the system down while not in use.

Note: the “Keep SOC on reset” is only available if the Bluetooth interface is running firmware v2.42 or higher! In case the Bluetooth interface firmware is below v2.42, please update the Bluetooth interface via VictronConnect on phone/tablet.

- Change the default setting for how to handle SOC on powerup to the new Keep SOC on reset feature. The other two options, start at 100% and start unsynchronized are also still available.

- Fix a problem where a reboot after a restore to default would clear the history. There was no  time limit between the reboot and the "restore to defaults" action to trigger this  issue: even after years a reboot would result in a loss of the history.

- Fix the problem of automatically overwriting an already configured “Charged Voltage” after completing the setup.

- Fix miscellaneous UI problems.

- Fix: immediately effectuate the "Backlight Always On" setting (50) when changed. Prior one had to wait for the timeout before the backlight switched off or had to manually press a button to make the backlight switch on.

- Fix: keep SOC unsynchronized after an update if it was already unsynchronized before the update (and "Keep SOC on reset" (70) is configured “Clear”).

- Fix showing a wrong “Relay state” (13) immediately after startup of BMV. This issue could temporarily manifest itself until both "Relay minimum closed time" (14) and "Relay-off delay" (15) had expired.

- Don't change SOC when executing a "Restore Defaults", unless a non-bmv monitor mode was selected prior to restoring to default. In that case, the SOC will be reset to 100%

Known issue:

- When the Auxiliary input is set to “Temperature”, the temperature compensation is not 0.0 and there is no temperature sensor connected then the calculated state of charge (SOC) is incorrect. In extreme cases, this is noticeable as a SOC that is always 1%.

Workaround:

When no temperature sensor is used, set the Auxiliary input to “None”. This is also the default for this setting since v4.05 and “Starter Battery” for earlier versions.

v4.09 – 6 October 2021 (BMV-712, BMV-712 Rev2, BMV-710H)

- Added command to retrieve MCU id.

v4.08 – 12 July 2021 (BMV-712 only)

- DC Monitor sign changes for current and power. When a DC Monitor is selected a positive current is shown for the intuitive usecase. For a source it means a positive current/power will be shown  when the current direction is from source to system. For a load a positive current/power will be shown when the current direction is from system into load.

- Water Heater is added as DC Monitor option.

- Charging related history items not shown when DC Monitor is selected.

v4.07 – 12 May 2021 (BMV-712 only)

- Added monitor mode (VE_REG_BMV_MONITOR_MODE). Setting can be configured via settings menu (item 72 in BMV) or via the vreg. When a non-bmv mode is selected all charging related functionality will be disabled (consumed energy, SOC, charged cycles, midpoint auxillary input, charging relay mode etc) and this also applies for the history items. Disabled items will show as "---" in the VE.Direct textframe. Note that when the mode is changed the history is NOT deleted (it’s up to the user to delete the history). The "MON" text item has been added to the VE.Direct text frame to reflect the monitor mode.

v4.05 – 2 February 2021

- Fixed possible continuing high pitch beep during the update.

- Fixed setting alarm/relay temperature below -17 Celcius.

- Fixed wrong relay activation due to using uninitialized voltage measurement at startup.

- Fixed returning wrong invalid value for VE_REG_DC_CHANNEL2_VOLTAGE when auxiliary input is not set to starter/aux (will return 0x7FFF now instead of 0xFFFF).

- Changed minimum accepted temperature for alarm/relay to -40 Celcius (was -99 Celcius).

- Limit maximum measured temperature to 100 Celcius. If the temperature  is exceeding this upper limit, the invalid value for VE_REG_BAT_TEMPERATURE is used (0xFFFF) and "---" for the "T" item in the text frame.

- Changed disabling automatic cycling of status items to “pressing minus button for 3 seconds” (same as enabling automatic cycling). While automatic cycling is enabled:

  - Pressing any button shortly will only enable the LCB backlight.

  - Pressing plus or minus shortly will show the previous/next status item.

  - Pressing setup button for 2 seconds will enable settings menu.

  - Pressing setup and select button simultaneously for 2 seconds will restore defaults.

- Fixed enabling automatic cycling of status items when not releasing button immediately after beep. Keeping minus button pressed when enabling automatic cycling will not cancel the automatic cycling anymore.

- Added "None" option to auxiliary input configuration. The "None" option is the default configuration. Settings 22-31, 39-48, 58-60, 67-68 are disabled in the settings menu (display) when the aux input is configured to "None".

v4.04 – 27 November 2019 (BMV710H only)

- First approved version for mass production BMV-710H

v4.03 – 5 September 2019 (BMV712 only)

- Change buzzer frequency from 8kHz to 4kHz. This is how it should have been from the beginning: equal to the BMV-700 & BMV-702.

v4.02 – 18 March 2019 (BMV712 only)

- Fixed production test.

Note: This version is released to production only; it is not available in VictronConnect or on

Victron Professional (DropBox).

Known issues:

- Buzzer frequency is at 8kHz, at the edge of what people can hear. This will be fixed in v4.03, September or October 2019. This bug was introduced in v4.01.

v4.01 – 31 October 2018 (BMV712 only)

- Added option for powering up in an unsynchronized state through setting #70 START SYNCHRONIZED (ON/OFF), ON by default. When set to OFF, the state-of-charge, time-to-go, and consumed energy are unknown until the first actual synchronization. Relay and alarm conditions based on these unknown parameters are disabled. The display’s CHARGE and (empty) battery indicator segments will blink.

- Battery indicator now scales between the configured discharge floor (Setting #16 “SOC Relay”) and 100% state-of-charge, instead of the full state-of-charge range (0% - 100%).

Note: the state-of-charge itself still scales between 0% and 100%:

- Introduced automatic cycling through status items. Enabled by holding the MINUS button for 3 seconds. Disabled by pressing any button.

- Fixed an issue related to changing the backlight intensity while connected to VictronConnect.

- Fixed an issue related to the maximum backlight intensity level.

- Internal changes that do not change the functional behavior

- Add options to disable Bluetooth: setting #71 BLUETOOTH MODE (ON/OFF)

Known issues:

- Buzzer frequency is at 8kHz, at the edge of what people can hear. This will be fixed in v4.03, September or October 2019. This bug was introduced in v4.01.

v4.00 – 19 June 2017 (BMV712 only)
First approved version for mass production.

Changelog BMV-7xx Smart Bluetooth Interface:

Note: the BMV-7xx Smart Bluetooth Interface can only be updated by VictronConnect (when connected via Bluetooth). As VictronConnect embeds the latest update, no separate update files are distributed for the BMV-7xx Smart Bluetooth Interface.

V2.52 – 11 November 2024

- Various small VE.Smart Networking fixes.

- Improvements on internal error handling and recovery.

V2.50 – 6 February 2024

- Small fix for recovering settings lock via PUK.

- Fix for not showing “Keep SOC on reset” option when no trend data is has been stored.

- Bootloader will be updated to v1.18

V2.47 – 26 September 2023

- Add support for “Settings lock” feature.

Note: the “Settings lock” is only available if the BMV is running firmware v4.15 or higher!

V2.46 – 19 September 2023

- Fix for occasional erroneous battery temperature broadcasts on VE.Smart networking, causing Smart Solars (on the same network) to stop charging.

- Fix for SOC trend scaling error. In some situations a wrong SOC value could end up in the stored trend for SOC (only the stored trend; the SOC value itself was not affected).

V2.43 – 23 January 2023

- Fix (rather rare) bug that could cause faulty temperatures to be transmitted in a VE.Smart network. This bug only affected BMVs without the (optional) temperature sensor. These “zombie” temperature broadcasts caused SmartSolars to stop charging for the rest of the day, to restart only the next day. Note that this bug was very rare to occur.

V2.42 – 5 September 2022

- Added new “Keep SOC on reset” feature. (!) Selectable in battery settings.

Note: the “Keep SOC on reset” is only available if the BMV is running firmware v4.12 or higher!

V2.41 – 25 May 2022

- Added support for Instant Readout feature

- Several trend stability fixes

- Bootloader will be updated to v1.16