Blue Smart IP22 Charger firmware change log

How to update?
This product can be updated using our VictronConnect app, via Bluetooth.

The VictronConnect app is available for phones, tablets and Windows and macOS laptops. Documentation & download.

What firmware version do I currently have?

To see the firmware version, look with the VictronConnect app

How to configure?

Use the VictronConnect app.

Change log:

v3.68 – 05 March 2026

Update for: Smart IP22 Charger, Rev2 only
Changes/features added:

- Fix new revision IP22 triple output models pulsing

- Improve new revision IP22 voltage control

- Fix no button lock feedback when in power supply mode

v3.67 – 30 January 2026

Update for: Smart IP22 Charger, Rev2 only
Changes/features added:

- Fixed an issue with some chargers not transitioning properly between bulk, absorption and float

- Fixed an issue for the IP22 single output chargers with some chargers experiencing large voltage/current spikes on the output at low charge currents (below approximately 5A)

v3.65 – 17 September 2025

Update for: Smart IP22 Charger, Rev2 only
Changes/features added:

- Add button lock feature. Requires VictronConnect v6.20 or later.

v3.63 – 11 April 2025

Update for: Smart IP22 Charger Rev2, single output only
Changes/features added:

- Fix new revision single output IP22 models, producing too high voltage under load

- Fix new revision single output IP22 models, unable to produce < 13.0V

v3.61 – 18 September 2024

Update for: Smart IP22 Charger, Rev2 only
Changes/features added:

- Fix low temperature cut-off

- Add Instant Readout

- Add synchronized charging to 230V models

- Time for fixed absorption can now be set to 0 on all chargers (disabling absorption)

- Repeated absorption can now be disabled on all chargers (requires Victron Connect update)

- Adaptive absorption time can now be set to 30 minutes on all chargers.

- Tail current can be configured on all chargers

- Update PUK entry procedure when 30 seconds BT is configured