# EV Charging Station firmware change log

# Product

The EV Charging Station has three-phase and single-phase capabilities. It delivers maximum 22kW AC in three-phase mode or 7.3kW in single-phase mode.

It incorporates 4.3-inch LCD touch screen for monitoring and control and a fully programmable RGB Light Ring around the charging port, to quickly determine the device state.

# How to update?

This product can be updated in two ways:

- Update remotely over the internet via our VRM Portal. It requires a GX device, such as a Color Control GX, Venus GX or Cerbo GX to be part of the installation.

- Update over the internal web interface (available from v1.22). Open internal web interface, go to Settings->Backup&FW->FW Update. Click the ‘Open’ button and select the '*.evcsup' file. Click the 'Update' button and wait until the update procedure finish.

Documentation for VRM: Remote firmware update is here.

# What firmware version do I currently have?

There are several ways to see the firmware version:

- Look on your GX device

- Look at the System-overview page on the VRM Portal

- Look on the left-bottom corner of the EV Charging Station LCD

- Look on the left-bottom corner of the EV Charging Station web interface

# How to configure?

EV Charging Station can be fully configured via built-in web interface. Please check this video tutorial.

# Change log:

v2.05 - 10 September 2025

Improvements:

- Minor stability and performance improvements

v2.04 - 2 September 2025

New features:

- Added "Online Updates" feature for firmware

Improvements:

- Improvements to meet cybersecurity requirements

- Improvements in Grid ON/OFF state detection

- Improvements in handling 'Start/Stop charging' based on backup battery SoC levels

Bug fixes:

- Fixed an issue causing a 2-second delay before power-off in the charged state

- Fixed incorrect Gateway address in Wi-Fi AP mode

v2.03 - 28 May 2025

Improvements:

- Reduced the voltage drop threshold for detecting a connected EV during the CP auto-calibration procedure

Bug fixes:

- Fixed an issue with 2s delay before power off in disconnected state

- Fixed an issue in the CP auto-calibration procedure that caused calibration to occur in a disconnected state

v2.02 - 22 April 2025

New features:

- Added CP line auto-calibration procedure. Activated by default

Improvements:

- Modified firmware version representation according to the common format

- Added retries for FW update over the web interface in case of error

- Added more warning messages related to display FW update

- Improvements for auto mode, related to start by low excess power and start by battery maximum SoC

Bug fixes:

- Fixed issue with special characters in WiFi SSID

- Fixed issue with display firmware storage

v2.1 - 13 January 2025

New features:

- Added light ring brightness limit option to Web interface and ModbusTCP server(register 5132)

Improvements:

- Improvements of the Web interface dashboard layout

- Improvements for display FW updating

- Forbidden start EV charging during display FW updating

- Removed notification from the display when the scheduled mode is not configured

Bug fixes:

- Fix GX communication issue related to UTC time registers(issue happened with old GX device FW)

- Fix the single power calibration issue

v2.0 - 21 October 2024

New features:

- The internal WEB interface was redesigned to a new version

- The display GUI was redesigned to a new version

- Added inverter overtemperature protection. If EVCS detects an overtemperature alarm on the inverter - reduced the charging current to the minimum value and shows the warning message

- Added a table for power calibration

- Added 'User' and 'Auto' types and 'GX' and 'WEB' sources for date\time synchronization

- Added additional options for 'Reboot to factory defaults'

- Added additional charging current limitation to meet German regulations

Improvements:

- The charging current max limit can be reduced to 6A

- Additional cases were added to start in auto mode if the backup battery is full and no PV production

- Auto mode 'Battery powered'. BKPB balance SOC max - default value will be 80%

- Modbus TCP server registers table was extended with new values(see EVCS-Modbus-TCP-register-list-v3.4.xlsx)

Bug fixes:

- Fix for BLE and iOS issues. There was a case when the iOS devices connected to EVCS but did not receive any data

- Fix for WiFi manual settings validation. There was an issue with the wrong subnetwork address calculations

- Fix for session data loss without EV reconnection

v1.30 - 2 September 2024

Bug fixes:

- Fix Auto mode start \ stop timeouts issue. On some systems, we observed a problem that EVCS wasn't starting charging in Auto mode and all the time was in a 'Low SoC' state. It was related to the start \ stop timeouts issue.

v1.29 - 8 July 2024

New features:

- Added a "Reboot" button to the web interface

- Added the "Allow low excess solar power for auto mode" option to the web interface

- Added the "Backup battery emulation" option to the web interface

Improvements:

- Improvements to increase configuration storing robustness

- Overload protection. Used different algorithms for AC-IN and AC-OUT connections. Added configurable 'Input current limit' option to the web interface for AC-IN connection

- Improvements for Auto mode timeouts control(start and current increase\decrease timeouts)

- Modbus TCP server registers table was extended with new values(see EVCS-Modbus-TCP-register-list-v3.2.xlsx)

- System performance and stability improvements

Bug fixes:

- Fix WIFI SSID min length

- Fix wrong charging start cases after the system reboots

v1.28 - 2 April 2024

New features:

- Added country\region code configuration during the ‘First setup’ procedure

- Added tamper detection based on the light sensor

Improvements:

- Improvements in WiFi module configuration and handling logic.

- Modbus TCP server registers table was extended with new values(see EVCS-Modbus-TCP-register-list-v3.1.xlsx)

- System performance and stability improvements

Bug fixes:

- Fix the light ring issue, when the light ring is OFF on some models of EVCS

- Fix the display issue, when the display shows not correctly painted layout

v1.27 - 6 November 2023

New features:

- Added ‘Date & Time’ configurations. Daylight saving offset will be calculated automatically based on the selected region and time zone. Added also  ‘Date & Time’ to the 'First setup' procedure

Improvements:

- Scheduler scene intersection checking is performed only if the scene is enabled. So, users can make a configuration first and then decide which scenes to enable\disable

- Added warning for scheduled mode if local date\time is not synchronized

- When GX data becomes outdated - EVCS will not modify the charging current in auto mode. In this case current will be the same until we not receive new data from GX or understand that GX become disconnected

- Added additional case to start charging if no PV power and the backup battery has max SoC

- The 'Start SoC' parameter has no effect anymore to stop charging in Auto mode

- Overload protection also reads an 'Overload' state from the inverter. The overload warning remains visible until the car is reconnected

- During stop charging the current value is reduced to the minimum

- FW update performance and stability improvements

- System performance and stability improvements

Bug fixes:

- Fix the incorrect sending of the actual data to the VictronConnect APP

- Fix the incorrect MB-TCP Server behavior in case of multiple client connections

v1.26 – 17 July 2023

New features:

- Added ‘Overload protection’ feature

- Added ‘Allow battery/grid power for auto mode’ feature

- Integrated all user-settings and ‘First-use setup’ with VictronConnect APP(requires a new VictronConnect APP version that is not publicly available yet)

- Added Warning/Error/Notification codes

- Added warning message when internal settings changed

- Added ‘External lock’ feature for grid operators

- Added current limitation to minimum value and delay before all relays off

Improvements:

- Added 'back off' algorithm for the next start try for auto mode

- Increased MB-TCP Server holding registers quantity to 200

- Added all user and internal settings to MB-TCP Server holding registers

- Added all user and internal settings to shell menu

- Main and display firmware updating processes optimizations

- 'Discover’ using only IP addresses with the same subnet

- Improvements for ON/OFF grid detection algorithm

- Improvements for PV detection algorithm and visualization on the main page

Bug fixes:

- Fix ‘Discover’ procedure lock timeout

v1.25 – 11 April 2023

New features:

- Added 'Battery powered' auto mode algorithm. Used by default

- Added grid failing processing(it will enable charging back if was active)

- Added pairing between EVCS and GX device by VRM Portal ID

- Added GX device communication check button in general settings

- Added Bluetooth disabling from the Victron Connect application

Improvements:

- Improved GX device communication and error handling. Added additional messages to the web interface in case of communication problems

- System performance improvements

v1.24 – 26 January 2023

New features:

- Added Bluetooth support and VictronConnect integration

- Added scheduler for charging

Improvements:

- Minor web interface improvements

- System performance improvements

- Increased speed of the display FW updating

Bug fixes:

- Fix display wake-up event

v1.23 – 20 September 2022

New features:

- Added CP line calibration procedure

- Added Power calibration procedure

- Added ‘Contactor active when charged’ option

- Added configurable backlight levels for display

- Added ‘Hide Wi-Fi credentials’ option for display

- Added ‘Minimum charging current’ option

Improvements:

- Redesigned display GUI. System schematic become updatable. Reduced display FW size.

- Modified 'main FW' and 'display FW' version representation on the web interface and display

Bug fixes:

- Fix WIFI SCAN procedure(on SCAN error was not possible to scan other channels)

- Fix charger state processing on the web interface

v1.22 – 5 July 2022

New features:

- Added configurable IPs whitelist for MB-TCP-Server

- Added total energy counter

- Added FW update over local web interface

Improvements:

- Change default device name to ‘EVCS-SN’

- Change favicon.ico

- After the 'Fist setup' procedure is finished - QR code always has the address of the Victron site, otherwise - local AP address

- Rename all counters names on the main Web page

- Added error codes for the FW-Update procedure

- ‘Settings->ModbusTCP Client’ renamed to ‘GX device’

- If communication with ‘GX device’ is disabled - ‘EV charger position’ will always be like ‘Inverter AC in’

- Increased MB-TCP-Server registers quantity to 100

- MB-TCP-Server. Added backward compatibility for registers quantity

- Increased WiFi password max length to 63 characters

- Recalibrated curent measurement sensors

- Improved Login\Logout procedure

- Added ‘exit’ command for shell

Bug fixes:

- Fix SSH server autentification

- Fix shell ‘backspace’ key processing

- Fix shell commands history processing

- Fix response processing during display FW update

- Fix commands processing from the display when ‘Control via display’ is disabled

- Fix ON\OFF Grid state detection

- Fix GX devices list appearance

- Fix car images on the internal web interface

- Fix internal web interface main page layout

- Fix stability issues in web app communication

v1.21 – 04 April 2022

First approved version for mass production.