# EV Charge Station V2 firmware change log

# Product

The EV Charge Station V2 has three-phase and single-phase capabilities. It delivers maximum 22kW AC in three-phase mode or 7.3kW in single-phase mode.

It incorporates a 4.3-inch LCD touch screen for monitoring and control and a fully programmable RGB Light Ring around the charging port, to quickly determine the device state.

# How to update?

This product can be updated in three different ways:

- Update remotely over the internet via our VRM Portal. It requires a GX device, such as a Color Control GX, Venus GX or Cerbo GX to be part of the installation.

- Update via the internal web interface. Open the internal web interface and go to Settings->FW Update.

For an online update, open the ‘Online updates’ tab and click the ‘Press to check’ button. The device will check whether a new firmware version is available. If a new version is found, click ‘Press to update to’ to start the update.

To update using a firmware file, open the ‘Offline updates’ tab. Click ‘Open’ and select the ‘*.evcsup’ firmware file. Then click ‘Update’ and wait until the update procedure is complete.

The ‘*.evcsup’ firmware files can be downloaded from the Victron Professional website.

- Update via the VictronConnect app. Connect to your device using the VictronConnect app. If a new firmware version is available, the app will display a dedicated update notification. Follow the instructions in the app to perform the firmware update.

Documentation for VRM: Remote firmware update is here.

# What firmware version do I currently have?

There are several ways to see the firmware version:

- Look at the System-overview page on the VRM Portal

- Look on the left-bottom corner of the EV Charging Station LCD

- Look on the left-bottom corner of the EV Charging Station web interface

# How to configure?

EV Charge Station V2 can be fully configured via built-in web interface or VictronConnect app. Please check this video tutorial.

# Change log:

v1.02 - 19 October 2026

New features:

- Extended the First-setup procedure for VictronConnect app to match the functionality available in the web interface

Improvements:

- System performance and stability improvements

v1.01 - 5 October 2026

First approved version for mass production