Ion Control Firmware change log

Update using VE Power Setup. This program is available for download on https://www.victronenergy.com/software/.

Special hardware, a USB to Canbus interface, is also necessary. The Victron CANUSB, part number ASS030532000, can be used. VE Power Setup is also compatible with other interfaces, such as Actisense and Kvaser.

To see what version you currently have: scroll to the Diagnostics screen, and look at the “Ion Control” version.

Change log:

v1.08 - 22 September 2014

Changes:

- Added support for the two new models: Lynx Ion + Shunt 350A and 600A

v1.07 - 22 July 2013

Changes:

- SOC and TTG disappearing has been fixed

- Supports both 2600 and 2700 type hardware

- Last 4 known errors added to diagnostic page (requires Lynx Ion v1.18 or higher)

- Shut down due to error counter added to diagnostic page (requires Lynx Ion v1.20 or higher)

- Min/max cell voltage added to history page (requires Lynx Ion v1.20 or higher)

- Charge/Discharge renamed to Allow-to-charge and Allow-to-discharge

- Bilge pump output removed

- Backlight intensity setting improved

v1.06 - 28 November 2012

Temporary release to fix on the problems with SOC/TTG.

Changes:

- Workaround for SOC/TTG displaying bug (the display restarts every 2 hrs)

- Reset device by pressing button 1 and 5 (outer most) simultaneously

v1.05 - 16 October 2012

Changes:

- Removed ‘Number of Cycles’ from History screen. The number of cycles readout can be very confusing, since cycles are only counted when state of charge rises and falls below strict limits. It is much more accurate to use the Cumulative Amp-hours as an ‘age-meter’.

Known issues:

- SOC and TTG value disappear after some time. All other values (V, A, Ah) remain visible. To show them again, reset the display by keeping the left two buttons pressed. Do not press them too long, because then you will end up in boot-loader mode. If you are in boot-loader mode, power cycle the display (unplug and plug the connector at the back) to get it back online.

v1.04 - 23 August 2012 – last stable version

Changes:

- Both known issues from v1.03 fixed.

v1.03 - 1 August 2012

Changes:

- Added battery voltage to Battery n screens

- Fixed maximum cell voltage update issue on the Battery System screen

Known issues:

- Does not report software version to VUP. Power on and scroll to Diagnostics screen to see the version.

- Does not restart automatically after a software update via VUP. It needs to be power cycled in order to start the software after updating.

v1.02 - 27 July 2012

Changes:

- Changed N2K model to "Ion Control"

- Removed boot-loader version from Diagnostics screen

- Added Lynx Shunt software version to Diagnostics screen

- Fixed issue with BMS error flags in Lynx Ion Status screen

- Changed Balancing values to active/inactive instead of enabled/disabled

- Added Battery System screen, displaying min/max cell voltage and battery configuration

- Added Lynx Ion IO screen, displaying the status of the contacts on the Lynx Ion

- Added Battery n screens, displaying the temperature and cell voltages per battery

v1.01 - 7 June 2012

Changes:

- Display N2K out of range value as "inf"

v1.00​ ​- 4 June 2012

Changes:

- initial release