Lynx Distributor changelog

How to update?

It’s not possible to update the firmware of the Lynx Distributor.

What version do I have?

Devices with serial numbers HQ18xxxxxx and below always have firmware v1.0.

Devices with firmware v2.0 (or later) have the firmware version added on the product and packing labels.

Change log:

V2.1 – 22 February 2019

- Bug solved that shows the wrong color of the power LED when no bus power is supplied.

V2.0 – 13 February 2019

- Fuse measurement method changed. Now a fuse is marked as blown when the voltage drop across the fuse >0.5V. With this method, the Lynx Distributor can not only be used to monitor the load fuses, but it’s also possible to monitor power input fuses.

- I2C interface added to communicate with future Lynx devices that are able to read the status of the fuses. It’s possible to connect (daisy-chain) up to 4 Lynx Distributors that all need their own unique address. The address can be set using the DIP-switches on the PCB.

The Lynx Distributors with firmware v2.0 can also be used together with older Lynx devices.

Prior releases are not documented here.