PPP2 firmware change log

Products:

PPP2 8Ah, 10Ah, 20Ah and 30Ah

How to update?

Update using VictronConnect. This program is available for download on http://www.victronenergy.com/software/. Special hardware, the VE.Direct to USB interface (part number ASS030530000), or the VE.Direct Bluetooth Smart dongle is required.

What firmware version do I currently have?

There are several ways to see the firmware version:

- Check the number written on the part- and serial number label on the back

- Look with VictronConnect

- Look on the Color Control GX.

- Look at the System-overview page on the VRM Portal.

How to configure:

Make sure to use the latest version of VictronConnect.

Note

In some special cases the PPP can lock-up when updating the firmware. In this case the update fails and VictronConnect is unable to recognize the PPP afterwards, while the red and blue LED on the PPP keeps blinking very fast. When you experience this problem, please contact your dealer.

Change log:

v1.08 – November 2016

- Improved charging when charger is connected for a long time

v1.07 – March 2016

- Fix problem that the PPP would not switch off when connected to a mover or other load with a high capacitance on its input.

v1.05 and v1.06 have not been released.

v1.04 – 2016 June 9

- Improve balancing by increasing charge voltages

- Increase reaction speed to an over-voltage event.

v1.02 and v1.03 have not been released.

v1.01 – March 2016

First released version