Smart BatteryProtect changelog

How to update?

Update using VictronConnect. This program is available for download on https://www.victronenergy.com/support-and-downloads/software.

What version do I have?

Firmware version is visible in VictronConnect.

Change log:

v2.11 – January 23, 2023

- Based on v2.10

- Added support for Instant Readout

  - VictronConnect can now show the most relevant Smart BatteryProtect data in the product list page without having to stablish a connection

- Added support for Random PIN codes

  - Disallowed product release in Test mode, when PIN not programmed

v2.10 – September 20, 2022

- Released for 48V-100A only

- Improved “Disabled SBP will now try to pull output to 0V” behavior (introduced in v2.07)

  - To decrease mosfets drop-out during production

v2.08 – March 22, 2021

- Added PROG option for PIN reset (‘P’)

  - Thus making it possible to reset PIN without the need of a PUK code

v2.07 – February 16, 2021

- Disabled SBP will now try to pull output to 0V

  - Thus providing reliable Phoenix Inverter REMOTE disconnect

- Several internal improvements

v2.05 – December 15, 2020

- Improved Bluetooth disconnect behavior

- Fixed spurious error E6 (Reference voltage failure)

- Several internal improvements

v2.04 – February 20, 2020

- Added Charge Protection, which prevents overheating that happened when the product is installed wrongly, ie. by connecting a charger (e.g. a Multi) to the output. Note that still, also with this firmware update, the product is not to be installed in such manner. It is not designed to handle reverse currents. All currents need to go from IN to OUT. Which also means it cannot be used in combination with an inverter/charger like Multis and Quattros. This protection merely reduces the risk of overheating, smoke and fire in case of such error in the installation- design or -wiring. 
In detail:

  - When (output is disabled and) output voltage is 0.5V higher than battery voltage and internal temperature is above 45 °C, the output will be enabled.
Output will be kept enabled until internal temperature drops below 45 °C for 30 seconds.
All this time E8 is blinking on the display and showing an error in VictronConnect.

- Support indication of default Bluetooth PIN.

- Support advertising connection state while being connected to VictronConnect.

- When changing the PIN, drop the existing bond information, such that user needs to pair again when reconnecting.

- Fix issue with VE.Smart, where communication could come to a halt.

- Several internal improvements.

v2.01 – November 8, 2018

- First approved version for mass production.

- Ships bootloader v1.02.

Prior releases are not documented here.