# VE.Direct LoRaWAN Modules Changelog

What version do I have?

You can find your version number in the VRM portal. Goto your installation -> open device list in the left menu -> unfold the gateway tab and find the firmware version.

How to update?

It is not possible to update this device.

Changelog:

## v1.05 – December 2018

Note: all units with versions earlier than v1.05 have been recalled for a free software upgrade; due to the issue that they stop transmitting after a while. How long it takes before they stop transmitting varies; and is typically between 12 and 60 days.

Changes:

- Complete the fixes of v1.04 and update base-firmware.

Models:

- ccfe6ed-1.7.1.b2-Trimm				EU Version

- ccfe6ed-1.15.0.b1				US Version

- ccfe6ed-1.15.0.b1				AS Version

- 4b4c8d7-1.15.0.b1				AU Version

## v1.04 (released only as AU)

Changes:

- Fix module no longer transmitting after 298 cycles; typically between 12 and 60 days after powerup.

- Fix various issues introduced in base-firmware v1.8 by upgrading to 1.15

- Remove the problematic channel 8 for the AU region.

Models:

- abcdefg-1.15.0.b1					AU Version - 10 units delivered

## v1.03

Changes:

- Added AS and AU region support

Models:

- 770439b-1.7.1.b2-Trimm 				EU Version

- 770439b-1.8.0.b1 				US Version

- 770439b-1.8.0.b1-ELECTRONET 			AS Initial release

- 770439b-1.8.0.b1 				AU Initial release

Known issues:

- Fix module no longer transmitting after 298 cycles; typically between 12 and 60 days after powerup.

- AU only: Channel 8 frequency causes the unit to crash and get stuck.

- 770439b-1.7.1.b2-Trimm: Error on boot. There is no work around; Victron will replace the unit under warranty.

## v1.02 (released only as US)

- Added US region support

- Improved internal logging mechanism.

5cd0bef-1.8.0.b1 				US Initial release

Known issues:

- Fix module no longer transmitting after 298 cycles; typically between 12 and 60 days after powerup.

## v1.01 – Initial release

f8d05d8-1.7.1.b2-Trimm 				EU Initial release

Known issues:

- Fix module no longer transmitting after 298 cycles; typically between 12 and 60 days after powerup.

- Error LED does not stay red for the full 10 seconds before a reboot.